National Snow and Ice Data Center. 1996. Arctic Ocean Snow

and Meteorological Observations from Drifting Stations:

1937, 1950-1991. Version 1.0. CD-ROM available from

nsidc@kryos.colorado.edu, NSIDC, University of Colorado

at Boulder.

For an overview of the documentation on this CD-ROM,

please refer to \document\0readme.txt. For more information

about the CD-ROM directory structure and filenames, see

\document\cd.txt. For more information about the data format

see \document\format.txt. For more information about the

GeoVu data browse software see \document\geovu.txt. You are

now reading:

Data Description

(\document\data.txt)

CONTENTS

I.  Introduction

II.  Combined Data (\combined\*.*)

III.  Meteorological Data (\met\*.*)

A.  Air Pressure

B.  Air and Snow Surface Temperature

C.  Air Temperature and Relative Humidity

D.  Wind Velocities and Directions

E.  Clouds

IV.  Position Data (\position\*.*)

V.  Precipitation Data (\precip\*.*)

VI.  Solar Radiation Data (\solar\*.*)

VII.  Snow Data (\snow\*.*)

A. Snow Lines

B. Snow Stakes

VIII.  Wind Data (\wind\*.*)

IX.  References

I. INTRODUCTION

This data description document is an edited summary of

documents provided to the University of Washington Polar

Science Center by the Arctic and Antarctic Research

Institute (AARI). For the combined data, the document

briefly describes the data processing procedures used to

merge the measured data into the combined (daily-averaged

data set. For each of the four types of measured data, this

document identifies the data parameters, and briefly

describes the collection methods and instruments. For a

complete description of the CD-ROM directory structure and

filenames, please refer to \document\cd.txt.

The data on this CD-ROM are high quality in-situ

observations from drifting ice stations in the Arctic Ocean,

taken from 1950 through 1991. The data on the CD-ROM

include:

- A combined file containing daily averages of all the

observations at each station. This file is in the top level

\combined\ directory. The data are ASCII text and are

visible to the GeoVu browse software. A description is

provided in section II of this document.

- Edited, merged, quality-checked and otherwise processed

files, at the original temporal and spatial resolution.

These files are in the top level directories as shown in the

contents list above as items III through VIII. These files

are all ASCII text, and all are visible to the GeoVu browse

software except \snow\measured\lwc.dat and

\snow\measured\density.dat. Descriptions of the edited and

merged files are provided in sections III through VIII of

this document. Information about GeoVu browse software is

provided in the file \document\geovu.txt.

- Quality-checked data in the original temporal and spatial

resolution, and in the format provided by AARI. These files

are in the top level directory \original\. The files are

ASCII text; they are not visible to the GeoVu browse

software. Descriptions of these data are the same as above,

in sections III through VIII of this document. File and

record format descriptions are provided in

\document\format.txt.

II. COMBINED DATA (\combined\*.*)

The combined data set consists of daily-averaged

meteorological data from the original three- or six-hourly

observations, merged with the daily observations of

precipitation, radiation, and snow depth. In addition to the

observed variables, each record in the data set contains the

station ID as well as the year, month, date, and latitude

and longitude of the observation. Latitude and longitude are

linearly interpolated from the original randomly-timed

position records, to obtain daily positions.

Many data fields in the combined data set contain missing

data flags (9999). The reason for the missing data is that

this combined.dat file contains radiation data for only NP-

16 through NP-31, while the meteorological data are

presented for NP-1 through NP-31.

III. METEOROLOGICAL DATA (\met\*.*)

The measured meteorological data on this CD-ROM include

three- or six-hourly and daily meteorological observations

of:

- air pressure (mb)

- air temperature (degrees C)

- relative humidity (%)

- surface winds, including wind speed (m/s)

and wind direction (degrees)

- clouds including total cloud amounts (tenths)and low

cloud amounts (tenths)

In general, meteorological observations were carried out

according to a unified method presented in "Directions to

meteorological stations and posts" (1985) using instruments

as described below for each meteorological parameter.

A. AIR PRESSURE

Pressures were measured by using two mercury barometers, one

as the main instrument, and the other as a control. Pressure

measurements from the control (calibration) barometer were

taken at regular intervals; readings from the main barometer

are provided here. Air pressure is given in KPa referred

to a temperature of 0 degrees C and standard gravity force,

i.e. gravity at a latitude of 45 degrees at sea level.

To measure atmospheric pressure, a station cistern mercury

barometer (SP-A) and an inspector's mercury siphon-cistern

barometer (IP) were used.

The main parameters of the SP-A are:

range of the instrument:              810-1070 KPa

maximum error:                        < or = 0.5 KPa

accuracy of reading:                  0.1 KPa

scale spacing of cosine:              0.1 KPa

scale spacing:                        1.0 KPa

range of the barometric thermometer:  -5 - +45 degrees C

error of the barometric thermometer:  0.5 degrees C

The main parameters of the IP are:

range of the instrument:             570-1070 KPa

maximum error:                       0.3 KPa

accuracy of reading:                 0.05 KPa

scale spacing of cosine:             0.05 KPa

scale spacing:                       1.0 KPa

range of the barometer thermometer:  -5 - +45 degrees C

B. AIR AND SNOW SURFACE TEMPERATURE

Above 30 degrees C, air and snow surface temperature were

measured using mercury thermometers. Below 30 degrees C,

alcohol thermometers were used. Starting with NP-21, the

temperature of dry and wet-bulb thermometers as well as the

temperature of the snow/ice cover were measured using wire

resistance thermometers recorded electronically.

Measurements of snow-ice cover temperature were carried out

only in the winter. Because of intensive insolation,

repeated diurnal freezing and thawing cycles, and pronounced

spatial heterogeneity of the surface in summer (melting

snow, melted ice, ponds), such measurements cannot be

considered representative.

In order to measure the temperature above -30 degrees C and

to determine the relative humidity, dry and wet-bulb

meteorological psychrometric thermometers (TM-4) were used

in a "gilled" shelter (similar to a Stevenson's Screen).

The main parameters of the TM-4 are:

range of the instrument:  -35 to +40 degrees C

scale spacing:            0.2 degrees C

measurement error over the range from

+0C to +50C:  < or = 0.2 degrees C

< 0C:  < or = 0.3 degrees C

At air temperatures below -30 degrees C, alcohol

meteorological thermometers TM-9 were used in order to

measure the air as well as the snow-ice surface temperature.

The main parameters of the TM-9 are:

range of the instrument:  -65 to +25 degrees C

scale spacing:            0.5 degrees C

measurement error over the range from

+20 to -20C:  0.5 degrees C

-30 to -40C:  0.8 degrees C

-40 to -50C:  1.0 degrees C

-50 to -60C:  1.5 degrees C

Starting with NP-21, copper wire resistance thermometers

(MT-102) were used as the wet-bulb thermometer as well as to

measure the air and snow-ice surface temperature.

The main parameters of the MT-102 are:

range of the instrument:  -60 - +50 degrees C

temperature resistance

coefficient:  430x10 -5x10W/degrees C

rated resistance of the

thermometer (at 0C):  2501 W

delay(s) in the air

at ambient velocity of 2m/s:  < or = 40

measurement error

(including recorder uncertainty):  +0.2 degrees C

C. AIR TEMPERATURE AND RELATIVE HUMIDITY

Air temperature and relative humidity were measured in

barometric booths (Stevenson Screens) with natural

ventilation at a height of 2 m.

Relative humidity (f) at a temperature above -10 degrees C

was determined by psychrometry, which is based on measuring

air temperature and the temperature of a wet-bulb

thermometer, i.e. the temperature of thermodynamic

equilibrium between expenditures of heat of evaporation from

the moistened surface and the head inflow to the thermometer

from the environment.

Values of f were calculated using the psychrometric table

(Table 1) below.

The results obtained using psychrometry were compared with a

hair hygrometer in order to check and correct the hygrometer

reading at regular intervals at air temperatures above -10

degrees C.

At the temperature above -10C, the psychrometric pair of the

dry and wet-bulb thermometers TM-4 was used in order to

measure f. The preliminary error estimate of f is given in

Table 1.

Table 1.  Measurement Range and Error of Relative Air

Humidity by Means of the Psychrometric Pair

Air            Measurement Error as a Percentage of the

Temperature    Measured Value of Relative Humidity (%)

(degrees C)

100       80        20        10

-----------    ---------------------------------------

10          3        4         11        20

0          4        6         17        35

-5          5        9         25        50

-10          7        12        35        70

At air temperatures below -10 degrees C, the model MV-1

meteorological hygrometer was used to measure relative air

humidity.

The main parameters of the MV-1 are:

range of the instrument:       30 - 100%

measurement error:             up to 10%

scale spacing:                 1%

operating temperature range:  -50 - +55 degrees C

D. WIND VELOCITIES AND DIRECTIONS

The most significant changes during the operating period of

the North Pole (NP) drifting stations were in methods and

means for measurement of wind velocity and direction, which

vary from the rudimentary hand-held anemometers with "make-

it-yourself" wind direction indicators to the modern

recording instruments used since NP-22. Also, the height of

installation of the wind velocity sensors has varied from 2

to 11 m. The anemometer M63-M was used beginning with NP-23

through NP-31.

Table 2 lists the instrument numbers and heights of

installation of the wind velocity sensors used for the

measurement of wind velocity and direction. Table 3 provides

the main parameters of the instrument. The years of

operation for each drifting station are listed in

\document\project.txt.

Table 2.  Instruments and Installation Heights Used for

the Measurement of Wind Velocity and Direction

Station   Instrument(s)       Height (Dates)

--------------------------------------------

NP-1      MS-13               2 m

NP-2      8I0001M             8 m

NP-3      MS-13; 8I0001M      8 m

NP-4      8I0001M; FVL        8 m

NP-5      FVL                 6 m

NP-6      FVL; FVT            8 m

NP-7      FVL; FVT           10 m

NP-8      FVL; FVT            8 m

NP-9      FVL; FVT            8 m

NP-10     M-49                6 m

NP-11     FVL; FVT; M-47      8 m

NP-12     M-49               10 m

NP-13     M-47; M12           6 m

NP-14     M-47;M-63           8 m

NP-15     M-63                6 m

NP-16     M-49               10 m

NP-17     M-63               10 m

NP-18     M-49; M-12          6.4 m  (11/68 - 05/69)

8.5 m  (05/69 - 10/71)

NP-19     M-64; M63M          6.4 m  (12/69 - 10/70)

7.5 m  (11/70 - 01/71)

9.5 m  (02/71 - 11/72)

12.0 m (11/72 - 03/73)

NP-20     M-47                8 m    (05/70 - 04/71)

6 m    (05/71 - 05/72)

NP-21     M-47               10 m

NP-22     M-64; M-63M        10 m

NP-23     M-63M              10 m

NP-24     M-63M              10 m

NP-25     M-63M              10 m

NP-26     M-63M              11 m

NP-27     M-63M              10 m

NP-28     M-63M              10 m

NP-29     M-63M              10 m

NP-30     M-63M              10 m

NP-31     M-63M              10 m

Table 3.  Main Parameters of the Instruments Used for

the Measurement of Wind Velocity and Direction

Threshold

Sensitivity

Range  Range      Measurement       for speed &

Speed  Direction  Speed Direction   direction

Instrument  (m/s)   (degree)   (m/s)   (degree)    (m/s)

----------   -----  ---------  -----    --------  ----------

MS-13     1-20      -    (0.3+0.06V)    -        0l.8/-

810001M   1-15    0-360      1.0       10        1.0/1.0

FVL   1-20    0-360      *          *          *

(at 16 compass points)

FVT   1-40    0-350      *          *          *

M-49   1.5-59  0-360  (0.5+0.05V)   10        1.2/1.2

M-47   1.5-50  0-360  (0.5+0.05V)   10        1.2/1.2

M-64   1.0-4.0 0-360  (0.5+0.05V)   10        0.8/1.5

M-12   1.0-4.0 0-360  (0.5+0.05V)   11.25     0.7/0.9

(at 16 compass points)

M-63   1.0-4.0 0-360  (0.5+0.05V)   10        0.8/1.5

M-63M   1.0-4.0 0-360  (0.5+0.05V)   10        0.6/1.0

Notes for Table 3:

1.  MS-13 measures only wind velocity.

2.  Measurement area of wind vanes with light and heavy

boards (FVL, FVT) depends on the observer's experience.

The error is not less than +/- 1 meter per second for

velocity and +/- 0.5 point (11 degrees) for direction.

3.  M=12, M-47, M-64, M-63, M-63M allow registering the wind

velocity averaged over 10 minutes; MS-13 allows

registering velocity averaged over 2-10 minutes.

4.  V in formulas for velocity measurement area is the

measured wind velocity, m/s.

E. CLOUDS

Quantitative characteristics of cloudiness (total and low

cloudiness) and the form of clouds were determined according

to the recommendations of "Directions to meteorological

stations and posts" (1985), and Makhover and Nudel'man

(1987). The quantity of clouds (cloudiness) was defined by

the total amount of sky covered by clouds, estimated in

tenths of coverage.

IV. POSITION DATA (\position\*.*)

Positions of the drifting stations were determined mostly by

means of terrestrial astronomical observations. Sun fixes

and star observations were made daily as season and weather

permitted. Positioning errors are estimated to have been

3-5 km. Starting with North Pole station NP-25, positions

were determined using satellite technology. NP-31 used the

Satellite Navigation system. AARI reviewed all position data

for this project using the original meteorological records.

The position data files received from AARI contained only

observations that were actually made. They were delivered by

AARI in files separate from the meteorological observations.

The position data were in Moscow time, but were converted to

UMT (GMT) at the Polar Science Center. Positions for day 360

of 1988 through day 60 of 1989 were missing from the NP-31

record. This gap has been filled by linear interpolation.

In the \combined\, \met\, \solar\, and \snow\averaged\

directories, these observed positions have been interpolated

after some quality control to provide an approximate

position for each meteorological observation. A simple

linear interpolation scheme was used between reported

observations in the three- and six-hourly files in the

directory \original\position\. The positions in

\met\metnp*.dat and in the radiation data files are

interpolated. Interpolated positions were used in the

combined data file of daily averaged data,

combined\combined.dat.

V. PRECIPITATION DATA (\precip\*.*)

Precipitation was measured using a shielded rain gauge

mounted on a geodesic frame. The upper section of the bucket

was 2 m high (Somov, 1954). This method was used for all the

North Pole stations, NP-1 through NP-31.

The precipitation data are amount (mm) and type (solid,

mixed, rain).

VI. SOLAR RADIATION DATA

(\solar\*.* and \original\solar\*.*)

The radiation data for stations NP-17 through NP-31 are

presented with the meteorological variables for those same

stations, for the convenience of the user, in the file

\solar\dailyavg.dat. These daily-averaged solar radiation

measurements are also included in the combined data set

\combined\combined.dat. The large combined data set has many

"missing data" flags because it includes the

meteorological and snow data for all stations, NP-1 through

NP-31.

The daily-averaged solar radiation data are also presented

in tabular form in the directory \original\solar\. Each

table has a header containing year, month, station number,

variable name and units. The variables and associated

filenames are:

- diffuse radiation (W/m^2)      \original\solar\spd*.tab

- direct radiation (W/m^2)       \original\solar\sps*.tab

- global radiation (W/m^2)       \original\solar\spg*.tab

- reflected radiation (W/m^2)    \original\solar\spr*.tab

- albedo (%)                     \original\solar\spa*.tab

- net radiation (budget)( W/m^2) \original\solar\spb*.tab

The radiation data have been previously published in

Marshunova and Mishin (1994), which contains a complete

description of the measurement techniques and data

processing. The text is provided on this CD-ROM in ASCII

text format as \document\aari\handbook.txt. The published

data tables are also included on this CD-ROM

(\document\aari\marshun\tables\table*.txt).

Original solar radiation units were Mj*(m^-2)*(day^-1)*100

for all values except albedo, which is in percent. A

conversion factor of 8.64-1 was applied to the radiation

data in the combined data set, so that the data could be

provided in the more commonly used units of W/m^2.

VII. SNOW DATA (\snow\*.*)

The snow data on this CD-ROM include:

- surface temperature (degrees C)

- snow depths along snow lines (cm)

- snow depths at stakes (cm)

- density (g/cm^3)

- liquid water content (mm)

Regular snow measurements were begun in 1953 at NP-3.

Consistent techniques were used at all stations, NP-3

through NP-31. A complete description of the snow data is

provided in Radionov et al., 1997), a draft translation of

which is provided in \document\aari\radionov.txt. Radionov

et al. is also in press at Gidrometeoizdat, St. Petersburg,

Russia. A translation is in progress at the University of

Washington, which will be published as a University of

Washington Applied Physics Laboratory (APL) Technical

Report.

A. SNOW LINES (\snow\measured\snowline.dat)

Snow depths were measured every 10 m along a line 500 to

1000 m long when the depth was at least 5 cm and more than

50% of the surrounding area was snow covered. Direction of

the line was chosen randomly, and deviated from a straight

line only to avoid hummocks. Subsequent snow lines were

measured 3 m from the previous line. Bad data are flagged by

-99. Averages of the snow lines are provided in the

directory \snow\averaged\.

Snow densities and liquid water content (i.e. snow water

equivalent) were also measured along each line, but only

every 100 m. These data are provided in

\snow\measured\density.dat and \snow\measured\lwc.dat.

B. SNOW STAKES (\snow\measured\snwstake.dat)

Near the NP stations, about 100 m from the nearest building,

daily measurements of snow depth were made at three corners

of a 25 m square area marked by stakes. This file contains

the average of the three measurements. Months having fewer

than 31 days show "666" as placeholders in the data fields.

"999" is the missing data flag. The original "three stake"

data can be found in \original\snwstake\np*.*

VIII. WIND DATA (\wind\np.wuv)

These files are the same as \met\metnp_**.dat except the

wind data are expressed by U and V components instead of as

wind speed and wind direction.

The wind data were decomposed into U and V components from

rectangular coordinates, using the procedure developed by

the Arctic Ocean Buoy Program (now International Arctic Buoy

Program, IABP) at the University of Washington. The equation

showing this conversion is:

r = reported wind speed

theta = reported wind direction

phi = (longitude - theta)*3.1415927/180

U-component of wind = r*cos(phi)

V component of wind = r*sin(phi)

Reference:  Thorndike, A. S. and R. Colony. 1983. Objective

analysis of atmospheric pressure and sea ice motion over the

Arctic Ocean. In: Thorndike, A. S., R. Colony and E. Mun~oz,

Arctic Ocean Buoy Program Data Report, 1 January 1982 - 31

December 1982. Polar Science Center, University of

Washington, p. 123-132.

IX. REFERENCES

Directions to meteorological stations and posts. 1985.

Iss. 3. Part 1. Leningrad: Gidrometeoizdat.

Makhover, Z. M., and L. A. Nudel'man. 1987. The Aviation-

Climatological Characteristics of the Northern Hemisphere

(Aviatsionno-klimaticheskie kharakteristiki severnovo

polusharia), Volume 1: Cloudiness, Moscow, Gidrometeoizdat.

Marshunova, M. S., and A. A. Mishin. 1994. Handbook

of the radiation regime of the Arctic Basin: Results from

the Drift Stations. V. F. Radionov and R. Colony, eds.,

University of Washington Applied Physics Laboratory

Technical Report APL-UW TR9413, 52 pp. plus appendices.

Radionov, V. F., N. N. Bryazgin, and E. I. Aleksandrov.

1997. Snow cover of the Arctic Basin. L. M. Ctrovich, ed.,

St. Petersburg, Gidrometeoizdat, in press. English

translation by I. Solovyova and T. C. Grenfell.

Somov, M. M., ed. 1954-55. Observations of the drifting

research station of  1950-1951 (Materialy nablyudeniy

nauchno-issledovatel'skoy drayfushchey stantsii, 1950-51

goda.) Arkticheskiy Nauchno-Issledovatel'skiy Institut,

Leningrad, Vol. III, Section 8, p. 6. American

Meteorological Society translation.

This file is one of six documentation files for this CD-

ROM. The other five are:

0readme.txt   provides an overview of the CD-ROM content;

includes pointers to documentation files and

instructions for citing use of the data in

publications

cd.txt   describes the directory structure and

filenames on the CD-ROM

format.txt   describes the record format of each type

of file on the CD-ROM

geovu.txt   describes the GeoVu menu-driven data display

and visualization software, how to obtain

it via ftp and install it on your computer;

also describes the basic GeoVu commands

project.txt   describes the history of the Russian North

Pole Drifting Stations, the utility and

relevance of the station observations, and

the collaboration to rescue the data;

includes the project acknowledgments

For assistance or additional information, please contact:

NSIDC User Services       E-mail: nsidc@kryos.colorado.edu

CIRES, Campus Box 449     Phone:  303-492-6199

University of Colorado    Fax:    303-492-2468

Boulder, CO 80309-0449 USA